ARDUINO CODE: SMART LOCKER

#include <WiFi.h>

#include <Firebase_ESP_Client.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Keypad.h>

#include <ESP32Servo.h>

#include <DHT.h>

#include <SPI.h>

#include <MFRC522.h>

// WIFI

#define WIFI_SSID "Abi"

#define WIFI_PASSWORD "Abinayak"

// FIREBASE

#define API_KEY "AIzaSyBt3Vapz2NYYFXcU24Zf57sUmubYbw3qOY"

#define DATABASE_URL "https://smartlocker-519a0-default-rtdb.asia-southeast1.firebasedatabase.app/"

#define DATABASE_SECRET "Ztn98JkVXfwx09HgvWDJmekP2OiPCCFtVEyDNb7W"

FirebaseData fbdo;

FirebaseAuth auth;

FirebaseConfig config;

// LCD

LiquidCrystal_I2C lcd(0x27, 16, 2);

// SERVO

Servo doorServo;

#define SERVO_PIN 4

// VIBRATION

#define VIB_PIN 34

// ALERT

#define ALERT_PIN 16

// DHT22

#define DHTPIN 15

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// MQ2

#define MQ2_PIN 35

// RFID

#define SS_PIN 5

#define RST_PIN 17

MFRC522 rfid(SS_PIN, RST_PIN);

// KEYPAD

const byte ROWS = 4, COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[ROWS] = {13, 12, 14, 27};

byte colPins[COLS] = {26, 25, 33, 32};

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

// PASSWORD

String correctPassword = "*6B4";

String input = "";

// VARIABLES

int attempts = 0;

String vibrationStatus = "SAFE";

String lockStatus = "LOCKED";

bool rfidAccess = false;

float temperature = 0;

float humidity = 0;

int gasValue = 0;

// 🔥 vibration count variables

int vibrationCount = 0;

unsigned long lastCheck = 0;

bool triggered = false;

// FIREBASE

void sendToFirebase() {

Firebase.RTDB.setInt(&fbdo, "/locker/attempts", attempts);

Firebase.RTDB.setString(&fbdo, "/locker/vibration", vibrationStatus);

Firebase.RTDB.setString(&fbdo, "/locker/lock_status", lockStatus);

Firebase.RTDB.setFloat(&fbdo, "/locker/temperature", temperature);

Firebase.RTDB.setFloat(&fbdo, "/locker/humidity", humidity);

Firebase.RTDB.setInt(&fbdo, "/locker/gas", gasValue);

}

// SERVO

void openDoor() {

doorServo.write(90);

lockStatus = "UNLOCKED";

lcd.clear();

lcd.print("Door OPEN");

sendToFirebase();

delay(3000);

doorServo.write(0);

lockStatus = "LOCKED";

lcd.clear();

lcd.print("Door CLOSED");

sendToFirebase();

delay(2000);

rfidAccess = false;

lcd.clear();

lcd.print("Scan RFID");

}

void setup() {

Serial.begin(115200);

lcd.init();

lcd.backlight();

doorServo.attach(SERVO_PIN);

doorServo.write(0);

pinMode(VIB_PIN, INPUT);

pinMode(ALERT_PIN, OUTPUT);

dht.begin();

SPI.begin(18, 19, 23, 5);

rfid.PCD_Init();

// WIFI

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

lcd.print("Connecting WiFi");

while (WiFi.status() != WL_CONNECTED) delay(500);

lcd.clear();

lcd.print("WiFi Connected");

// FIREBASE

config.api_key = API_KEY;

config.database_url = DATABASE_URL;

config.signer.tokens.legacy_token = DATABASE_SECRET;

Firebase.begin(&config, &auth);

Firebase.reconnectWiFi(true);

delay(2000);

lcd.clear();

lcd.print("Scan RFID");

}

void loop() {

// RFID FIRST

if (!rfidAccess) {

if (rfid.PICC_IsNewCardPresent() && rfid.PICC_ReadCardSerial()) {

Serial.println("RFID Scanned");

lcd.clear();

lcd.print("RFID Scanned");

delay(1000);

lcd.clear();

lcd.print("Enter Password");

rfidAccess = true;

}

}

// 🔥 VIBRATION COUNT LOGIC

int vib = digitalRead(VIB_PIN);

if (vib == HIGH) {

vibrationCount++;

}

// check every 1 second

if (millis() - lastCheck > 1000) {

Serial.print("Vibration Count: ");

Serial.println(vibrationCount);

if (vibrationCount > 5 && !triggered) {

vibrationStatus = "DETECTED";

Serial.println("⚠ STRONG VIBRATION!");

digitalWrite(ALERT_PIN, HIGH);

delay(300);

digitalWrite(ALERT_PIN, LOW);

sendToFirebase();

triggered = true;

}

else if (vibrationCount <= 2 && triggered) {

vibrationStatus = "SAFE";

Serial.println("✅ VIBRATION NORMAL");

sendToFirebase();

triggered = false;

}

vibrationCount = 0;

lastCheck = millis();

}

// DHT22

float t = dht.readTemperature();

float h = dht.readHumidity();

if (!isnan(t) && !isnan(h)) {

temperature = t;

humidity = h;

}

Serial.print("Temp: "); Serial.print(temperature);

Serial.print(" Hum: "); Serial.println(humidity);

// GAS

gasValue = analogRead(MQ2_PIN);

Serial.print("Gas: "); Serial.println(gasValue);

// KEYPAD

if (rfidAccess) {

char key = keypad.getKey();

if (key) {

input += key;

lcd.setCursor(0, 1);

lcd.print("                ");

lcd.setCursor(0, 1);

lcd.print(input);

if (input.length() == 4) {

if (input == correctPassword) {

lcd.clear();

lcd.print("Access Granted");

Serial.println("Access Granted");

attempts = 0;

delay(1000);

openDoor();

}

else {

lcd.clear();

lcd.print("Wrong Password");

Serial.println("Wrong Password");

attempts++;

sendToFirebase();

delay(1500);

lcd.clear();

lcd.print("Enter Password");

}

input = "";

}

}

}

delay(100);

}